# PART 1 - GENERAL

1.01	SUMMARY

1.	This section includes Section 155.5 of the Regulations of the New York State Commissioner of Education “Uniform Safety Standards for School Construction and Maintenance Projects” for maintaining a Certificate of Occupancy during construction.

1.02	COMPLIANCE AND REGULATIONS

1.	The Contractor shall comply with the requirements of the various codes referred to in these specifications.  Comply with regulations in effect on the date Contract Documents are issued, unless otherwise indicated.  All work shall conform to and be performed in strict accordance with all governing agencies such as, but not limited to:

.1	New York State Uniform Fire Prevention and Building Code

.2	New York State Education Department

.3	New York State Department of Labor

.4	New York State Department of Environmental Conservation

(DEC)

.5	Occupational Safety and Health Administration (OSHA)

.6	United States Environmental Protection Agency (EPA)

.7	National Fire Protection Association - NFPA.101, 2006.

.8	New York State Energy Conservation Construction Code, 2010.

.9	Americans with Disabilities Act 1992.

.10	ICC/ANSI A117.1 -2010 For Handicapped Access.

.11	Asbestos Hazard Emergency Response Act.

.12	Guidelines for the Evaluation and Control of Lead-Based Paint

Hazards in Housing (HUD)

.13	City of Rochester Regulations (Within Street Right of Way

portions of project)

1.03	PERMITS AND FEES

- Permits;  The Contractor shall obtain and pay for all permits, fees, licenses, certificates, inspections, and other use charges required in connection with the work.  Generally, the Building Permit is obtained at no cost by the District directly from the New York State Education Department.  Applicable Contractors shall obtain and pay for all costs for all other required fees, permits, inspections, etc. (such as street cut fees, electrical inspections, etc.).

2.	Street Work;  All work within a street right-of-way shall comply with

all local City of Rochester (County or State) and related regulations,

requirements, etc.

3.	Utility Work;  All work that includes connections, modifications, etc. of utility services, to comply with respective utility regulations, requirements, etc.

4.	All work on sewer systems tying into public sewers shall comply with Monroe County Pure Waters Regulations, requirements, etc.

1.04    GENERAL SAFETY AND SECURITY STANDARDS

1.	The occupied portion of any school building shall always comply with the minimum requirements necessary to maintain a certificate of

occupancy.

2.	Comply with the following general safety and security standards for construction projects.

1.	All construction materials shall be stored in a safe and secure

manner.

2.	Fences around construction supplies or debris shall be

maintained.

3.	Gates shall always be locked unless a worker is in attendance

to prevent unauthorized entry.

4.	During exterior renovation work, overhead protection shall be

provided for any sidewalks or areas immediately beneath the

work site or such areas shall be fenced off and provided with

warning signs to prevent entry.

5.	Workers shall be required to wear RCSD photo-identification

badges at all times for identification and security purposes

while working on District property.

1.05   SEPARATION OF CONSTRUCTION AREAS FROM OCCUPIED AREAS

1.  	Construction areas which are under the control of a contractor and therefore not occupied by District staff or students shall be separated from occupied areas.

1.	Provisions shall be made to prevent the passage of dust and contaminants into occupied parts of the building.  Periodic inspection and repairs of the containment barriers must be made to prevent exposure to dust or contaminates.

2.	Gypsum board must be used in exit ways or other areas that require fire rated separation.

3.	Heavy duty plastic sheeting may be used only for a vapor, fine dust or air infiltration barrier, and shall not be used to separate occupied spaces from construction areas.

2.	When working in occupied buildings, the Project Inspector will designate a specific stairwell and/or elevator for construction workers' use during work hours.

1.	In general, workers may not use corridors, stairs or elevators designated for students or school staff.

3.	Large amounts of debris must be removed by using enclosed chutes or a similar sealed system.

1.	There shall be no movement of debris through halls of occupied spaces of the building.

2.	No material shall be dropped or thrown outside the walls of the building.

4.	All occupied parts of the building affected by renovation activity shall be cleaned at the close of each work day.  School buildings occupied during a construction project shall maintain required health, safety and educational capabilities at all times classes are in session.

1.06   MAINTAINING EXITING AND VENTILATION DURING CONSTRUCTION

1.	An attached plan, if relevant, details how exiting required by the applicable building code will be maintained during construction, shall be complied with.  The plan indicates temporary construction required to isolate construction equipment, materials, people, dust, fumes, odors and noises during the construction period.  Temporary details shall meet code required fire ratings for separation and corridor enclosure.  At a minimum, required exits, temporary stairs, ramps, exit signs and door hardware shall be provided at all times.

2.	An attached plan, if relevant, details how adequate ventilation will be maintained during construction.  The plan will indicate ductwork that must be rerouted, disconnected, or capped in order to prevent contaminants from the construction area from entering the occupied areas of the building.  The plan will also indicate how required ventilation to occupied spaces affected by the construction will be maintained during the project.

1.07	FIRE HAZARD PREVENTION

1.	No smoking is allowed on District property, including construction areas.

2.	During construction, daily inspections of District occupied areas shall be conducted by District personnel to assure that construction materials, equipment or debris do not block fire exits or emergency egress windows.

3.	Proper operation of fire extinguishers, fire alarm, and smoke/fire detection systems shall be maintained through-out the project.

1.08	NOISE ABATEMENT DURING CONSTRUCTION

1.  	Construction and maintenance operations shall not produce noise in excess of 60 dba in occupied spaces or shall be scheduled for times when the building or affected building spaces are not occupied or acoustical abatement measures shall be taken.

2.	Noise level measurements (dba) shall be taken with a type 2 sound level meter in the occupied space in a location closest to the source of noise.

1.09   CONTROL OF CHEMICAL FUMES, GASSES AND OTHER CONTAMINATES

DURING CONSTRUCTION

1.    The contractor shall be responsible for the control of chemical fumes,

gases, and other contaminates produced by, including but not limited to, welding, gasoline or diesel engines, roofing, paving, painting, etc. to ensure they do not enter occupied portions of the building or air intakes.

1.	Fume Generation;  The Contractor shall submit a plan indicated how and where welding, gasoline engine, roofing, paving, painting or other fumes will be exhausted from the work site.  Contractors shall provide all temporary means to assure that fresh air intakes do not draw in such fumes.

2.	Off-Gassing Period;  The one week clean-up period in the Anticipated Construction Schedule is also provided as time for “off-gassing” of volatile organic compounds (VOC) introduced during construction.  Specific attention is warranted for activities including glues, adhesives, paint, furniture, carpeting, wall coverings, and drapery.  Manufacturers shall be contacted too obtain information regarding appropriate temperatures and times needed to cure or ventilate the product during use and before safe occupancy of the space can be assured.  The Contractor shall include the above-mentioned information and shall clearly highlight the information, as part of the shop drawing submittals.

1.	Building materials or furnishings which “off-gas” chemical fumes, gasses or other contaminants shall be aired out in a well ventilated, heated warehouse before it is brought to the project for installation, or the manufacturer’s recommended “off-gassing” must be scheduled between installation and use of the space.

3.	Burn-Off Period;  Heating and ventilation equipment requiring a “burn-off” period shall be run during the one week clean-up period in the Anticipated Construction Schedule to accommo-date the “off-gassing” of new heat exchangers, heating elements, burners, ventilation equipment, etc.

4.	Material Safety Data Sheets (MSDS);  The Contractor shall maintain all manufacturer’s MSDS sheets at the site for all products used in the project.  Copies of the MSDS sheets shall be given to the Architect/Engineer and the Project Inspector.  MSDS sheets shall be provided to anyone who requests them.

1.10   SAFETY AND HAZARD AWARNESS

1.	Security Identification;  The Contractors and subcontractors who will be on the project site are required to wear a photographic identification card provided by the District.  This card will bear the name of the employee and contractor.  Access to the project site shall be supervised by the Project Inspector at the project location.  Access will be denied if proper identification is not produced upon request.

2.	Construction Outreach Safety Program;  It is the responsibility of all prime Contractors to provide assurance to the District that his/her employees and his/her subcontractors’ employees have received the OSHA 10 hour Construction Outreach Safety Training, including Hazardous Awareness Training.  Submit certification of all workers prior to award of contract indicating compliance with referenced awareness training.  Certification must be dated within five (5) years of submittal.  The Contractor shall also submit documentation prior to award of a Health/Safety/Environment (HSE) Program approved by the Rochester Safety Council or similar agency.

3.	AHERA Report Review;  Prior to the commencement of work, the Contractor shall submit a signed statement to the Project Inspector, on the Contractor’s corporate letterhead, identifying the project name, stating to the effect that he/she has reviewed all documents pertaining to the identification of Asbestos Containing Building Materials (ACBM) at the project site(s) including the AHERA Plan and that all his/her workers and subcontractors have been informed and reviewed these documents and have received training to recognize and respond in accordance with applicable codes, regulations and contract documents when encountering ACBM.

1.11	HAZARDOUS BUILDING MATERIALS

1. 	Large and small asbestos abatement projects as defined by

12NYCRR56 shall not be performed while the building is occupied.

2.	If surfaces that will be disturbed by reconstruction have tested

positive as Asbestos Containing Building Materials (ACBM), such

remediation, encapsulation, modification or removal will be specified

herein, under Division 2.

END OF SECTION