# PART 1- GENERAL

1.1 	RELATED DOCUMENTS

1.   	Drawings and general provisions of Contract, including General Conditions and Division 1 Specification sections, apply to work of this section.

1.2 	DESCRIPTION OF WORK

1.   	The extent of removal work is shown on drawings and includes:

1.	Each prime contractor shall be responsible for the removal of all

items specified, so noted or indicated on the drawings which

form a part of his contract.

2.	Cease work and immediately notify Project Inspector prior to

the disturbance or removal of unanticipated Asbestos Containing

Building Materials (ACBM).  Refer to school records for locations

of known ACBM; in accordance with AHERA, a Management

Plan is available for public review at the administrative office of

every District building.

3.    Cease work and immediately notify Project Inspector prior to

disturbance or removal of known lead containing building

material.  Refer to current or on-going records for location

of known lead containing building materials.

1.3 	SUBMITTALS

1.   	Submit proposed methods and operations of building demolition to

Project Inspector for review prior to start of work.  Include in

schedule coordination for shut-off, capping and continuation of utility services as required.

1.4 	JOB CONDITIONS

1.   	Items of salvable value to Contractor may be removed from structure

as work progresses.  	Salvaged items must be transported from site

as they are removed.

2.   	Maintain existing utilities indicated to remain, keep in service, and

protect against	damage.

1.   	Do not interrupt existing utilities serving occupied or used

facilities, except when authorized in writing by authorities

having jurisdiction.  Provide temporary services during

interruptions to existing utilities, as acceptable to governing 	                        	authorities.

3.   	Promptly repair damages caused to adjacent facilities by removal

operations at no cost to the District.

PART 2 - PRODUCTS (Not applicable)

# PART 3 - EXECUTION

3.1 	DEMOLITION

1.   	Remove concrete and masonry in small sections.

2.	Properly protect persons and property at all times against harm

or damage of any kind during demolition.  Protect existing

construction from damage resulting from demolitions.  Any

damage to existing property resulting from demolition shall be

repaired immediately at the Contractor’s expense.  Execute all

demolition in such manner as to create a minimum of

interference with the use of the building, roads, streets, walks

and other adjacent facilities.

3.	Remove present construction carefully in order not to damage the

remaining construction scheduled to remain and to minimize amount

of restoration.  Support remaining construction adequately and

maintain supports until new supports are erected.  Keep dust and dirt

to a minimum; dampen all debris during demolition.  Remove debris

and salvage materials immediately to an offsite location except as

otherwise directed.

4.	Prior to commencement of work, the contractor(s) effecting such

systems shall survey all building electrical systems and components,

including fire alarm, intrusion, communications, clock and computer;

make written notice to the Project Inspector regarding existing

damages, missing items and incomplete systems.  Prior to the

conclusion of this project, the contractor shall verify with the Project

Inspector that all building systems have been returned to their

original conditions.

5.	Do not demolish or remove present construction materials or

present equipment until the replacement materials or equipment are

on hand and are ready to be installed in full and proper operating

condition except as otherwise indicated.  Exceptions may be permitted

with the written approval of the Architect/Engineer or as dictated by

scope of project.

6.	Provide temporary barricades, partitions, shoring and support as

required for protection of property and occupants.  Do not block or

obstruct means of egress or other passages required to remain open

either for reasons of safety, security or operation of the District.

7.	Provide poly barriers with "Z" flaps at all work locations to contain

dust to area of	construction.  The contractor is responsible to provide

positive ventilation of the work area.

8.	Protect all existing surfaces and finishes scheduled to remain.

Provide hardboard protection on floors throughout the entire work

area.  Provide walk mats at room entrances to prevent widespread

tracking of dirt.

9.	Provide enclosed chutes when debris is removed from elevated floor

levels.  Line all	openings of existing construction used as part of

chute location.

10.	Do not close or obstruct streets, walks or other occupied or used

facilities without documented written permission from authorities

having jurisdiction.

11.	Maintain existing utilities indicated to remain, keep in service and

protect against damage during removal operation.

12.	Verify existing heights, vertical locations, etc. prior to removal 	to

guarantee alignment between new and existing conditions.

3.2 	DISPOSAL OF REMOVED MATERIALS

1.   	The Contractor shall properly dispose of all removed items and debris

except as directed by the Project Inspector.  The District reserves

the first rights of refusal for lighting fixtures, plumbing fixtures and all other salvageable building components.  These items shall be

disposed of by the Contractor responsible for their removal unless

directed otherwise by the Project Inspector at the time of removal.

2.	All removed items that are noted on the drawings - “deliver to

District”, “deliver to the Plant Maintenance Shop”, except as

otherwise directed, shall be delivered to the Plant Maintenance Shop,

at the City School District Service Center, 835 Hudson Ave. Obtain a

receipt for proof of delivery.  Notify the Project Inspector who will

arrange a suitable time.

3.	Items that are noted on the drawings for re-installation prior to

completion of work shall be handled, stored and re-installed in

location indicated and operate consistent with use and performance

prior to temporary removal.  Damage to equipment and products

resulting from improper performance of the Contractor shall be

repaired at the Contractor’s expense.

4.	Unless otherwise noted, the District will be responsible for removing movable furnishings scheduled for disposal and previously relocated by the contractor.

5.	All other removed items shall belong to the Contractor responsible for

their removal.	This Contractor shall remove all such items from the

premises promptly.  Dispose of all removed items and debris in

accordance with all applicable Federal, State and	Local regulation

including but not limited to the NYSDEC.

## END OF SECTION