# PART 1 - GENERAL

1.01	PROGRESS CLEANING

1.	Cleaning;  During the construction period the Contractor is responsible for keeping the school building and grounds free from accumulations of waste materials and rubbish.  At weekly intervals the Contractor shall remove all discarded materials, packaging, scraps, and debris to avoid buildup of such material.  This includes minimally broom and mop cleaning all areas affected by construction and areas elsewhere in the building where tracking has occurred, or as directed by the District Inspector.

1.	Where multiple Contractors contributed to the common dust and debris, all Contractors shall contribute proportionately to broom and mop cleaning, or as directed by the District Inspector.  Access control may be instituted by the District Inspector as required to control tracking throughout the building.

2.	Occupied Space;  Within spaces being worked on while the building is occupied, the Contractor shall in addition to the above, maintain open and accessible spaces and daily clean and damp mop all dust and debris, mop all spills, and stains resulting from the work so that occupants may properly and normally function in the space and not track dust and dirt around.

3.	Dumpster;  The Contractor shall provide and maintain a dumpster appropriate to the needs of the Contractor’s work.  No Contractor shall utilize City School District dumpsters.

1.	Where multiple Contractors are employed on the project each Contractor shall provide his/her own dumpster.

4.	Disposal;  Remove waste materials from the site in accordance with applicable laws.  Comply with safety standards and governing regulations for cleaning operations.  Do not burn or bury waste materials at the site, or discharge volatile or other harmful or dangerous materials into the drainage system.

5.	Prior to Final Cleaning;  One week prior to the commencement of the school in session period, the Contractor shall cease work leaving status of renovations in an acceptably complete condition and provide restoration of work area and other effected building and site locations so as to allow occupancy and room set-up.  This shall minimally include;

1.	Removal of all debris which shall include boxes, packaging, wrappers, rubbish, stockpiled materials, etc.

2.	Removal of all remaining equipment, tools, gang boxes, ladders, materials not yet installed, etc. off site.  Alternate areas including trailers on site, or specific limited consolidated areas within the building may be approved by the District Inspector.

3.	Except as otherwise indicated or requested by the District Inspector or Architect/Engineer, remove temporary protection devices and facilities which were installed during course of the work to protect previously completed work during remainder of construction period.

4.	Restoration of construction not yet finished to a safe, clean state acceptable for occupancy and used by the occupants as applicable.  Provide temporary closures, finishes and equipment etc. as necessary.

5.	Cleaning which shall include wet mopping of all surfaces, including high out of sight areas such as window molds and ledges, tops of doors, tops of cabinets, etc.

6.	Expenses incurred on the District to complete the above work as a result of the Contractor’s inadequate cleaning.  Cleaning expenses incurred by the District as a result of Contractor’s neglect will be back-charged to the responsible Contractor at twice the cost.

7.	All dust and debris generated after this time shall be cleaned up on a daily basis and fully controlled and isolated.

1.02	FINAL CLEANING

1.	Final Cleaning;  Provide final cleaning of this projects’ work, including related areas impacted by this project, at time indicated, consisting of cleaning each surface or unit of work to normal “clean” condition expected for a first-class building cleaning and maintenance program. Comply with manufacturer’s instructions for cleaning operations.  The following are examples, but not by way of limitation, of cleaning levels required;

1.	Remove labels which are not required as permanent labels.

2.	Clean transparent materials, including mirror and window/door glass, to a polished condition, remove substances which are noticeable as vision obscuring materials.  Replace broken glass and damaged transparent materials.

3.	Clean exposed exterior and interior hard-surface finishes, to a dirt free condition, free of dust, stains, films and similar noticeable distracting substances.  Except as otherwise indicated, avoid disturbance of natural weathering of exterior surfaces.  Restore reflective surfaces to original condition.

4.	Remove debris and surface dust from limited-access spaces including roofs, plenums, shafts, trenches, equipment vaults, manholes, attics and similar spaces.

5.	Clean concrete floors in non-occupied spaces broom-clean.

6.	Clean hard-surface floors in occupied spaces wet mop clean.

7.	Vacuum clean carpeted surfaces and other similar soft surfaces.

8.	Clean plumbing fixtures to a sanitary condition, free of stains including those resulting from water exposure.

9.	Clean light fixtures and lamps as to function with full efficiency.

10.	Clean project site (yard and grounds), including landscape development areas, of litter and foreign substances.

11.	Sweep paved areas to a broom-clean condition, remove stains, petro-chemical spills and other foreign deposits.  Rake grounds which are neither planted or paved, to a smooth even textured surface.

12.	Wipe surfaces of mechanical and electrical equipment clean, remove excess lubrication and other substances.

13.	Prevent the spread of debris by providing plastic barriers between clean spaces and those still undergoing work or cleaning operations.

END OF SECTION